Exp No:16 SERIAL INTERRUPT USING UART (KEYBOARD INPUT) IN KEIL

AIM:

To write and simulate an Embedded C program using serial interrupt in 8051 microcontroller such that while a message is continuously transmitted, any key pressed from the keyboard generates an interrupt and is displayed through UART.

APPARATUS / SOFTWARE REQUIRED:

• PC with Windows OS

• Keil µVision IDE

• 8051 Simulator (Keil built-in)

• Serial Window (UART #1)

THEORY:

The 8051 microcontroller provides a serial interrupt that occurs when:

• RI (Receive Interrupt flag) is set after receiving a character, or

• TI (Transmit Interrupt flag) is set after transmission.

In this experiment:

• UART is configured in Mode 1 (8-bit UART)

• Timer1 is used to generate baud rate (9600)

• Serial interrupt is enabled using ES bit

• When a key is typed in the Keil Serial Window, the received character sets RI

• The Serial Interrupt Service Routine (ISR) is executed immediately This demonstrates interrupt-driven serial communication, which is more efficient than polling.

PROGRAM:

#include <reg51.h>

#include <stdio.h>

void serial_ISR(void) interrupt 4

{

char ch;

if (RI)

{

RI = 0;          // Clear receive interrupt flag

ch = SBUF;       // Read received character

printf("\n>>> INTERRUPT RECEIVED: ");

printf("%c\n", ch);

}

}

void delay(void)

{

unsigned int i;

for(i = 0; i < 50000; i++);   // Delay for visibility

}

void main(void)

{

SCON = 0x50;     // UART mode 1, 8-bit, REN enabled

TMOD = 0x20;     // Timer1 mode 2 (auto-reload)

TH1  = 0xFD;     // Baud rate 9600

TR1  = 1;        // Start Timer1

IE = 0x90;       // EA = 1, ES = 1 (enable serial interrupt)

TI = 1;

while(1)

{

printf("Hello World\n");

delay();

}

}

OUTPUT:

- Displaying a Message:

- During interruption:

OBSERVATION:

INTERPRETATION:

• Continuous “Hello World” printing indicates main program execution

• Typing a key sets the RI flag

• Setting RI invokes the serial interrupt

• ISR executes immediately and prints the received character

• Interrupt output appears between normal program outputs

• This proves that the interrupt pre-empts normal execution

RESULT:

Thus, the serial interrupt using UART was successfully implemented and verified in Keil simulator. The interrupt was triggered upon keyboard input and serviced correctly using an Interrupt Service Routine.